Project 1: Optimizing the Performance of a Pipelined Processor

520021910863	Heda Chen

520030910287	Xinming Shu	Xin_ming Shu@sjtu.edu.cn

- Introduction

Part A

What we need to do in this part is write three simple assembly programs to implement three functions in example.c in directory sim/misc. These programs are functionally equivalent to the C. We also annotate the code detailedly. The code is accurate, efficient and readable.

Part B

In Part B, we modify the hcl file of the Y86’s sequential design to add the new instruction iaddl. We add instruction code for iaddl instruction, then add dependency relations of iaddl to all boolean signals and design the datapath for iaddl such as generate control signals for src and dst.

Part C

In Part C, we implement the iaddq instruction in pipe-full.hcl, and then optimize the efficiency of the assembler file ncopy.ys and the file pipe-full.hcl. We try several methods such as applying loop unrolling and modifying the pipeline design in branch prediction for certain code pattern, which optimizes the performance and brings CPE down to 9.17.

Contribution

Heda Chen: finished Part A, B and C

Xinming Shu: retry Part A, B and C and finished the report

- Experiments

  - Part A

    - Analysis

This part is a ‘warm-up’ session of this project. Write and simulate the functions of the three functions in example.c using the Y86 directive. Then use YAS to compile and use YIS to run.

- Key Points

- Protect the callee-saved register EBX, EDI, ESI.

- Pull the correct element from the stack.

- Difficult Points

1) Implement function recursion smartly and clearly.

2) To be a proficient in the use of Y86 instruction set.

- Core Technique

1) Code line by line referring to functions wrote by C language.

2) Mimicking C functions, division of functional areas with enough and clear label.

3) Draw a picture of stack to ensure the correctness.

    - Code

- sum.ys

1) Initialize the stack frame

2) Data definition for sample linked list according to the file project1

3) Define main and call sum_list: following the calling conventions, use %rdi to store the first argument

4) Implement sum_list: initialize and jump into loop

5) Implement loop and check

- rsum.ys

1) Initialize %esp, define data and main as rum.ys

2) Sum the linked list elements recursively

- copy.ys

Copy a block of words from one part of memory to another (non-overlapping region), computing the checksum Xor of all copied words

- Data section, source and destination block

- Copy src to dest and return Xor validation

    - Evaluation

- sum.ys

Figure 1 Part A_sum

The %eax register has the correct value which is the return value of the function 0xcba.

- rsum.ys

Figure 2 Part A_rsum

The %eax register has the correct value 0xcba.

- copy.ys

Figure 3 Part A_copy

- The %eax register has the correct value which is the return value of the function 0xcba.

- Values are written into the memory correctly as I labeled in the picture above.

  - Part B

    - Analysis

In this part, we should extend the SEQ processor to support instruction iaddl by modifying seq-full.hcl.

- Difficult Points

1) Understand the processing logic and take a good command of the syntax of HCL.

2) Judge whether iaddl should be added into the functions in stages.

- Steps

1) In Fetch Stage, add IIADDL in the choices regions of (bool) instr_valid, (bool) need_regid and (bool) need_valC.

2) In Decode Stage, when icode is IIADDL, srcB is from rB since the second operand of iaddl is a register, dstE (where the result from ALU is passed towards) is rB since ‘iaddl constant, rB’ means rB += constant (rB is updated).

3) In Execute Stage, add IIADDL in the choices region of (bool) set_cc since iaddl operation involves ALU operation which will set flags.

4) In Execute Stage, when icode is IIADDL, aluA (the first op) is valC (the constant in the instruction) since ‘iaddl constant, rB’ means the first op is the constant (valC).

5) In Execute Stage, when icode is IIADDL, aluB (the second op) is valB (the value of the second register that is read) for the same reason above.

    - Code

Figure 4 Stages of executing the instruction

- Combining irmovl, opl calculation process, through analysis, we get iaddl calculation process, and then we modify the code as below:

- Fetch Stage

Figure 5 seq-full_fetch

- Decode Stage

Figure 6 seq-full_decode

- Execute Stage

Figure 7 seq-full_execute

    - Evaluation

Build a new instance of the SEQ simulator (ssim) based on this HCL file, and then test it on a simple Y86 program.

- First, we run a simple program asumi.yo (testing iaddl), part of the content was omitted below

Figure 8 Run asumi in TTY mode

- Then we retesting the solution using the benchmark programs

Figure 9 Part B_Benchmark test

- Performing regression test

Figure 10 Part B_Regression test

- Test the implementation of iaddl

Figure 11 Part B_test iaddl

  - Part C

    - Analysis

In this part, we were asked to speed up the program ncopy.ys as much as possible by modifying the nocpy.ys and HCL.

- Add iaddl instruction to pipe-full.hcl

Add the use of instruction iaddl, modify the file pipe-full as what we did in the file seq-full in Part B.

- Use the instruction iaddl to optimize the code:

Figure 12 Count++ before substitution

Figure 13 Count++ after substitution

- Avoid load and use conflict:

When a load and use hazard occurs, it will waste a clock cycle, which can be avoided and utilized by reordering instructions. What’s more, we use two registers to store the variable val, loading them separately and ahead of time.

Figure 14 Load-use hazard

Figure 15 Deal with load-use hazard

- 7-way Loop Unrolling

We perform a technique named ‘loop unrolling’ to deal with much overhead in testing and updating procedure of loops. We do multiple loops for the part that > 6, and update the relevant data at once, to reduce the number of times we execute the add and jxx instructions.

- Search Tree for Remaining Elements

For large inputs, it’s better to unroll the loops for more ways. However, for small inputs, it is important to choose a good method to process the remainning elements. For the part that <= 6, the simplest way is to write another loop for them. But there is another way performs better, that is, jump to different position for different number of remaining ones. Since Y86 does not support relative jump instruction, we designed a search tree to get the correct jump destination for each possibility.

    - Code

- 7-Way Loop Unrolling

- xorl %eax,%eax    # count = 0;

- iaddl -6, %edx    # len <= 6?

- jle S06           # if so, goto S06

- L0:

- mrmovl (%ebx), %esi     # tmp0 = src[0]

- mrmovl 4(%ebx), %edi    # tmp1 = src[1]

- andl %esi, %esi         # if tmp0 <= 0

- rmmovl %esi, (%ecx)     # dst[0] = tmp0

- jle L1                  # skip increasing counter

- iaddl 1, %eax           # else counter++

- L1:

- mrmovl 8(%ebx), %esi    # tmp0 = src[2]

- andl %edi, %edi         # if tmp1 <= 0

- rmmovl %edi, 4(%ecx)    # dst[1] = tmp1

- jle L2                  # skip increasing counter

- iaddl 1, %eax           # else counter++

- L2:

- mrmovl 12(%ebx), %edi   # tmp1 = src[3]

- andl %esi, %esi         # if tmp0 <= 0

- rmmovl %esi, 8(%ecx)    # dst[2] = tmp0

- jle L3                  # skip increasing counter

- iaddl 1, %eax           # else counter++

- L3:

- mrmovl 16(%ebx), %esi   # tmp0 = src[4]

- andl %edi, %edi         # if tmp1 <= 0

- rmmovl %edi, 12(%ecx)   # dst[3] = tmp1

- jle L4                  # skip increasing counter

- iaddl 1, %eax           # else counter++

- L4:

- mrmovl 20(%ebx), %edi   # tmp1 = src[5]

- andl %esi, %esi         # if tmp0 <= 0

- rmmovl %esi, 16(%ecx)   # dst[4] = tmp0

- jle L5                  # skip increasing counter

- iaddl 1, %eax           # else counter++

- L5:

- mrmovl 24(%ebx), %esi   # tmp0 = src[6]

- andl %edi, %edi         # if tmp1 <= 0

- rmmovl %edi, 20(%ecx)   # dst[5] = tmp1

- jle L6                  # skip increasing counter

- iaddl 1, %eax           # else counter++

- L6:

- andl %esi, %esi         # if tmp0 <= 0

- rmmovl %esi, 24(%ecx)   # dst[6] = tmp0

- jle ChkCond             # skip increasing counter

- iaddl 1, %eax           # else counter++

- ChkCond:

- iaddl 28, %ebx          # src += 7 * sizeof(Byte)

- iaddl 28, %ecx          # dst += 7 * sizeof(Byte)

- iaddl -7, %edx          # len -= 7

- jg L0                   # if len > 0, goto L0

- Binary Search Tree for Finding the Number of Remaing Loops

- S06:              # len in (x-6: [0, 6] -> [-6, 0])

- iaddl 3, %edx	 # (x-6: [0, 6] -> [-6, 0]) => (x-3: [0, 6] -> [-3, 3])

- jg S46          # len-3 > 0, len in [4, 6]

- je R3           # len-3 == 0, len = 3

- S02:              # len in (x-3: [0, 2] -> [-3, -1])

- iaddl 2, %edx   # (x-3: [0, 2] -> [-3, -1]) => (x-1: [0, 2] -> [-1, 1])

- jl R0           # len-1 < 0, len = 0

- je R1           # len-1 == 0, len = 1

- jmp R2          # len-1 > 0, len = 2

- S46:             # len in (x-3: [4, 6] -> [1, 3])

- iaddl -2, %edx # (x-3: [4, 6] -> [1, 3]) => (x-5: [4, 6] -> [-1, 1])

- jl R4          # len-5 < 0, len = 4

- je R5          # len-5 == 0, len = 5

- Unrolling of Remaining Loops

- R6:

- mrmovl 20(%ebx), %esi   # tmp = src[6]

- andl %esi, %esi         # if tmp <= 0

- rmmovl %esi, 20(%ecx)   # dst[6] = tmp

- jle R5                  # skip increasing counter

- iaddl 1, %eax           # else counter++

- R5:

- mrmovl 16(%ebx), %esi   # tmp = src[5]

- andl %esi, %esi         # if tmp <= 0

- rmmovl %esi, 16(%ecx)   # dst[5] = tmp

- jle R4                  # skip increasing counter

- iaddl 1, %eax           # else counter++

- R4:

- mrmovl 12(%ebx), %esi   # tmp = src[4]

- andl %esi, %esi         # if tmp <= 0

- rmmovl %esi, 12(%ecx)   # dst[4] = tmp

- jle R3                  # skip increasing counter

- iaddl 1, %eax           # else counter++

- R3:

- mrmovl 8(%ebx), %esi    # tmp = src[3]

- andl %esi, %esi         # if tmp <= 0

- rmmovl %esi, 8(%ecx)    # dst[3] = tmp

- jle R2                  # skip increasing counter

- iaddl 1, %eax           # else counter++

- R2:

- mrmovl 4(%ebx), %esi    # tmp = src[2]

- andl %esi, %esi         # if tmp <= 0

- rmmovl %esi, 4(%ecx)    # dst[2] = tmp

- jle R1                  # skip increasing counter

- iaddl 1, %eax           # else counter++

- R1:

- mrmovl (%ebx), %esi     # tmp = src[1]

- andl %esi, %esi         # if tmp <= 0

- rmmovl %esi, (%ecx)     # dst[1] = tmp

- jle R0                  # skip increasing counter

- iaddl 1, %eax           # else counter++

- R0:

    - Evaluation

- Rebuild the simulator and the driver programs, test the solution in GUI mode on a small 4-element array and test your solution on a larger 63-element array.

Figure 16 Part C_Test the solution

Figure 17 Part C_test on a 4-element array

Figure 18 Part C_test on a 63-element array

- Testing the pipeline simulator on the benchmark programs

Figure 19 Part C_benchmark test

- Testing your pipeline simulator with extensive regression tests

Figure 20 Part C_regression test

Figure 21 Part C_regression test (with iaddl)

- Testing your code on a range of block lengths with the pipeline simulator

Figure 22 Part C_correctness test

- CPE test

Figure 23 CPE test

- Conclusion

  - Problems

[In this part you can list the obstacles you met during the project, and better add how you overcome them if you have made it.]

  - Achievements

[In this part you can list the strength of your project solution, like the performance improvement, coding readability, partner cooperation and so on. You can also write what you have learned if you like.]